| Operator Door can be opened. |  |
| --- | --- |
|  | DO: LP569 01 |

Open the Operator Door keeping the [S/G DOOR UNLOCK] switch pressed.

(Safety switch SL1632 (Operator Door unlock) is energized.)

DI: SL912 = 1

Operator Door cannot be opened as the following operations

are active.

- Spindle is operating.

- Axis feed being performed.

- Coolant pump is operating.

- Tool change is operating.

- Tool magazine is operating.

- Automatic operation being performed.

Pro. S

DI: LS912 = 0

DO: LP569 = 0

Above operations are completed.

The following operations are inhibited.

  - Spindle operation

  - Axis feed

  - Coolant pump operation

  - Tool change operation

  - Automatic operation

  - Cycle start and Mode Selection

  - One-touch functions

Close Operator Door.

DI: LS912 1 SL912  0

Pro. S detects Operator Door is closed.

Motor power supply and output signal common lines are enabled.

Machine operation can be performed.