Tool unclamping is commanded.

DO: SL01B 01 DO: SL01A 10

Solenoid valve SL01B (Tool Unclamp) is energized.

Hydraulic oil is supplied to rear of hydraulic cylinder and the piston moves forward.

DI: LS00 10

Piston comes into contact with rear of draw bar and draw bar compresses disk spiring.

Collet closes and release Tool Holder.

DI: LS601 10 DI: LS602 10

Draw bar pushes Tool Holder out of Spindle Taper.

Limit switch LS01 (Tool Unclamp) changes from “0” to “1”

DI: LS01 01

Tool unclamp completed.